RISK ASSESSMENT FOR AN ACTIVITY INVOLVING THE USE OF HAZARDOUS CHEMICALS

as required under the CONTROL OF SUBSTANCES HAZARDOUS TO HEALTH REGULATIONS (COSHH) and the DANGEROUS SUBSTANCES AND EXPLOSIVE ATMOSPHERES REGULATIONS (DSEAR)

This form aids in assessing the hazards associated with the use of chemicals and the subsequent application of suitable controls. Guidance on completion of the form can be found in Guidance Note 052: Risk Assessments for Hazardous Chemicals:

http://www3.imperial.ac.uk/safety/guidanceandadvice/chemicalsafety/hazardouschemicals

The sections below correspond approximately to the sections in the Guidance Note.

- The form does not address the risks associated with biological agents or ionising radiation – other Safety Department forms exist for this purpose.

- With respect to DSEAR, this form may be used to assess risks arising from small scale laboratory operations where hazard zoning is not usually applicable and the controls required largely mirror those of COSHH.

- This form is not suitable for assessing flammability and explosion risks arising from large scale operations (including pilot scale) where hazard zoning is necessary – this will require a more detailed assessment.

It is the responsibility of the person directing the research i.e. the Principal Investigator to ensure that risk assessments are carried out, remain valid and that the control measures identified are applied.

**Before undertaking workplace risk assessments, assessors should complete Risk Assessment Foundation Training (RAFT)**

| PERSON RESPONSIBLE FOR THIS WORK (THE PRINCIPAL INVESTIGATOR) |  |  |
| --- | --- | --- |
| Name: Dr. Zahid Durrani |  | Position: Reader |
| Department/ Section: EEE | Division: OSD | Faculty: Engineering |

| PERSON CONDUCTING THIS ASSESSMENT |  |
| --- | --- |
| Name: Dr. Zahid Durrani | Position: Reader |
| Department/ Section: EEE / OSD | Date risk assessment undertaken: 30 - 10 - 2016 |

| THE ACTIVITY or PROCEDURE |  |
| --- | --- |
| Title: Handling and use of Positive and Negative Photoresist | Assessment reference No: |
| Description (describe the procedure step by step or attach a Standard Operating Procedure): Use of small quantities of photoresist in a extracted setup for spinning thin films. 1. Photoresists both positive and negative are procured in litre bottles. 2. Photoresists are always handled and dispensed in extracted wet bench. 3. Photoresist is decanted into a small (e.g. 60 ml ) amber colour bottle for everyday use. 4. Photoresist is always spin coated in a dedicated extracted wet bench. 5. 2 to 3 ml of resist is dispensed on to a sample and spinned to spread and produce a thin film. 6. This film is baked on a hot plate or in an oven to dry off for UV exposure to transfer mask patteren into resist.. |  |

1.0 HAZARDS AND RISKS

1.1 HAZARD EVALUATION & RISK DETERMINATION

List each of the chemical substances used in the activity or procedure in the table below and use this matrix to determine the risk level for each one.

| (A) Health Hazard | (B) Dustiness or *Volatility | (C) **Quantity | Score |
| --- | --- | --- | --- |
| Low Hazard Statements: H303; 305 H313; 315; 316 H320 H336 EUH066 | Low Solids: Pellet-like solids that do not break up. Little or no dust observed during use. Solids forming large crystals Liquids: Boiling point > 150oC | Small <1g (ml) | 1 |
| Medium Hazard Statements: H301; H302 H311; H312; H314; H317 H318; H319 H332; H335 H371; H372; H373 | Medium Solids: Smaller crystalline or granular solids. Minimal dust, or if any dust is seen it settles out quickly. Liquids: Boiling point between 50 and 150oC | Medium 1 to 100g (ml) | 2 |
| High Hazard Statements: H300; H304 H310 H330; H331; H334 H340; H341 H350; H351 H360; H361; H362 H370 EUH070 | High Solids: Fine, light powders. Dust can be seen during use and possibly remains airborne for several minutes. Liquids: Boiling point < 50oC | Large > 100g (ml) | 3 |

Multiply (A)x(B)x(C) to estimate overall risk level:  ≤ 7 Low;   8-11 Medium;   ≥ 12 High

* if a chemical has more than one risk phrase, use the one(s) with the highest health hazard score

** When stating quantity, this should consider the quantity in the stock bottle as well as the quantity of the aliquots, since loss of containment from the entire stock bottle whilst removing aliquots may represent the greatest risk.

| Substance | Route of exposure | Hazard Statement(s)* (see safety data sheet) | ‡WEL | Health Hazard Score | Dustiness / Volatility Score | Quantity Score | Overall Risk Level L/M/H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inhalation (In) |  |  |  |  |  |  |
|  | Ingestion (Ig) |  |  |  |  |  |  |
|  | Skin contact (Sk) |  |  |  |  |  |  |
|  | Penetration (P) |  |  |  |  |  |  |
|  | Eye splash (Es) |  |  |  |  |  |  |
| Photoresist | Sk In | H319 |  | 2 | 2 | 2 | 8 |

‡Workplace Exposure Limit (if one has been assigned - see safety data sheet) – usually in ppm for vapours or mg/m3 for particulates.

1.2 PROCESS FACTORS INFLUENCING THE RISK OF EXPOSURE

| Are there any process factors that influence the route of exposure (See Section 2.2.9): |  |  |  |
| --- | --- | --- | --- |
| Weighing | Pipetting | Filtering | Elevated Temperature |
| Shaking / Mixing | Centrifugation | Use of Sharps | High Pressure |
| Other Decanting into small bottles and dispensing on to substrate for spinning |  |  |  |

1.3 IDENTIFICATION OF THOSE AT RISK OF EXPOSURE

Are there any substances listed in this activity having the Hazard Statements H360, 361 or 362 i.e. those affecting women of child-bearing age?

| Yes | No | If ‘Yes’: - any females working with the substance must be informed that the substance(s) present a reproductive toxicity hazard. - should they fall pregnant or are trying for pregnancy, then they have the option of contacting Occupational Health for a detailed confidential health assessment. - they should also avoid being involved in any large-scale spillage clean up in the event that such an incident arises. |
| --- | --- | --- |

Are there any substances listed in this activity having the Hazard Statements H340, 341, 350 or 351? (substances causing genetic defects or cancer)

| Yes | No | If ‘Yes’, this risk assessment once completed, must be submitted to the Safety Department (safetydept@imperial.ac.uk) for review before work is carried out. Information will be uploaded to a central database for record keeping purposes. |
| --- | --- | --- |

Are there any substances listed in this activity having the Hazard Statements H317 or 334 (skin or respiratory sensitisers) AND have been determined to be in the high risk category using the matrix above?

| Yes | No | If ‘Yes’, this risk assessment once completed, must be submitted to Occupational Health (occhealth@imperial.ac.uk) for review before work is carried out. |
| --- | --- | --- |

If ‘Yes’ is answered to either of the above question, indicate the frequency and duration of use

|  | Frequency (how often is the substance used) e.g. every day; once a month etc. |
| --- | --- |
|  | Duration (how long is possible exposure likely to be) e.g. five minutes; one hour etc. |

Are there any external factors that increase the risks associated with exposure to any of these substances e.g. contact lens wearing?

| Yes | No | If ‘Yes’ give details: |
| --- | --- | --- |

Are there any personnel other than laboratory workers who may be at risk from exposure? (e.g. maintenance workers, cleaners etc.)

| Yes | No | If ‘Yes’ give details: |
| --- | --- | --- |

1.4 SUBSTANCES SUBJECT TO OTHER LEGISLATION

Are there any substances involved in this activity that are subject to either the

Chemical Weapons Act or the Anti-terrorism, Crime and Security Act? You must keep a record of usage for Schedule 2 chemical weapons precursors as you will be contacted by the Safety Department on an annual basis prior to the College making its annual declaration to the Department of Energy and Climate Change.

| Yes | No | If ‘Yes’ give details: |
| --- | --- | --- |

Are there any substances involved in this activity that are listed by the Home Office as drugs precursors?

| Yes | No | If ‘Yes’ give details: |
| --- | --- | --- |

Are there any substances involved in this activity that are defined as controlled drugs?

| Yes | No | If ‘Yes’ give details: |
| --- | --- | --- |

Are there any substances involved in this activity that are explosive, flammable or oxidising? (See Q2.2.2e overleaf)

| Yes | No | If ‘Yes’ give details: |
| --- | --- | --- |

1.5 OTHER HAZARDS

Are there any other hazards involved with this activity? (e.g. pathogens, GMO’s, ionising radiation etc.) If ‘Yes’ have these risks been assessed and any necessary approvals obtained?

| Yes | No | If ‘Yes’ give details and / or reference numbers of related documentation: |
| --- | --- | --- |

2.0 CONTROL MEASURES

2.1 PREVENTION OF EXPOSURE

2.1.1 Elimination, substitution and procedural change

a). Can any of the substances be eliminated from the process?

| Yes | No | Give details: All handling and processing is carried out in extracted wet bench. |
| --- | --- | --- |

b). Can any of the substances be substituted by a safer alternative or a safer form of the same substance?

| Yes | No | Give details: |
| --- | --- | --- |

c). Can the method of work be changed so that the operation giving risk to exposure is no longer necessary?

| Yes | No | Give details: |
| --- | --- | --- |

d). Are measures in place to exclude non-essential personnel from the area?

| Yes | No | Give details: Swipe card entry for authorised personal only |
| --- | --- | --- |

2.2 CONTROL OF EXPOSURE

2.2.1 Minimising quantities

Can the quantities of the substances stored, used and produced as waste be reduced?

| Yes | No | If ‘Yes’ give details: already minimised |
| --- | --- | --- |

2.2.2 Containment and ventilation

a). Can some or all parts of the process be carried out on the open bench with good general ventilation? (if additional containment such as drip trays are required, give details)

| Yes | No | If ‘Yes’ give details: general good practice |
| --- | --- | --- |

b). Is a fume cupboard or other form of local exhaust ventilation required for any part of the process?

| Yes | No | If ‘Yes’ give details including type and location: Extracted wet bench in clean room 709 |
| --- | --- | --- |

c). Is the fume cupboard or other form of local exhaust ventilation subject to a maintenance regime?

| Yes | No | N/A | If ‘Yes’ give date of last test and state who is responsible for organising maintenance. March 2016 Facilities management |
| --- | --- | --- | --- |

d). Does any part of the process need to be totally enclosed e.g. inside a glove box?

| Yes | No | If ‘Yes’ give details: |
| --- | --- | --- |

e). Do measures need to be taken to control sources of ignition?

| Yes | No | If ‘Yes’ give details: |
| --- | --- | --- |

f). Is a chemical spill kit required?

| Yes | No | If ‘Yes’ give details: Material is used only in small quantities (e.g. 2-3 ml) at a time |
| --- | --- | --- |

2.2.3 Storage and transportation

a). Storage. If relevant, outline the storage arrangements:

| Toxic |  |
| --- | --- |
| Corrosive |  |
| Flammable / Highly Flammable / Extremely Flammable | Stored in small quantities in screw cap bottles, Bulk bottles are stored in Fridge. |
| Other |  |

b). Transportation on site. Will any of these substances need to be transported to other parts of the building or other parts of the site?

| Yes | No | If ‘Yes’ give details of how containment will be assured: |
| --- | --- | --- |

c). Transportation off site. Will any of these substances need to be transported to other sites?

| Yes | No | If ‘Yes’ give: | Name of other site: Substance and quantity: Method of transport proposed: Containment precautions: |  |
| --- | --- | --- | --- | --- |

2.2.4 Personal protective equipment (PPE)

| Gloves Type Disposible nitrile | Eye / face protection Type Safety Glasses |
| --- | --- |
| Respiratory protection Type | Clothing Type (in addition to lab coats) |
| State which parts of the process PPE is required for (e.g. experimental process or in emergencies only): good laboratory practice |  |

2.2.5 Waste disposal

Outline the disposal route for each substance or derivative

| Substance | Drain | Specialist waste contractor | Other (specify) |
| --- | --- | --- | --- |
| Photoresist |  |  | organic solvent waste |

2.2.6 Hygiene measures

Describe the hygiene measures in place for work involving these substances e.g. handwash facilities, laundering of protective clothing, storage of personal clothing, prohibition of eating and drinking etc.

Gloves all the time - handwashing - no eating / drinking, general good practice

2.2.7 Monitoring

Is monitoring necessary to validate the efficacy of control measures for any of these substances?

no

2.2.8 Information, instruction and training / supervision

Describe the information, instruction, training and supervision requirements for those working with these substances (include details of record keeping)

Completion of COSHH, general good practice

2.2.9 Lone working

Does any aspect of this activity present an overt risk to the lone worker due to any of the following factors?

|  |  | Details including additional precautions to be implemented |  | SOP/CoP/Protocol Reference: |
| --- | --- | --- | --- | --- |
| Can a lone worker safely enter and exit the facility? Including donning and removing PPE? | Yes No | If no, describe why and any additional precautions necessary for lone working: |  |  |
| Can any associated equipment be used safely by the lone worker? | Yes No | If no, describe why and any additional precautions necessary for lone working: |  |  |
| Does any substance used present a risk of rapidly incapacitating a lone worker? | Yes No | If yes, describe why and any additional precautions necessary for lone working: |  |  |
| Will a lone worker be able to safely deal with a spillage or other emergency e.g. malfunctioning equipment? | Yes No | If no, describe why and any additional precautions necessary for lone working: |  |  |
| Will lone work take place in a secure or restricted access facility? | Yes No | If yes, describe why and any additional precautions necessary for lone working: During normal working hours |  |  |
| Is lone working permitted for this activity? |  | Yes No |  |  |
| Is specific consent required for lone working? |  | ## Yes (each lone working activity) ## Yes (but not for each specific instance) Specific consent is not required for lone working |  |  |
| If lone working will take place, are there any restrictions as to when lone working can take place? |  | Yes No N/A | If yes or no, describe the restrictions and why these are in place: in labs and during normal working hours to prevent contamination and bad practice |  |

2.2.10 Additional controls

Are there any additional controls required in relation to the process factors identified in Section 1.2 that are not covered anywhere else? (http://www3.imperial.ac.uk/safety/guidanceandadvice/chemicalsafety/hazardouschemicals/hazchemcontrolmeasures)

| Process factor | Identified route of exposure | Description of how exposure is controlled |
| --- | --- | --- |
| Weighing |  |  |
| Pipetting |  |  |
| Filtering |  |  |
| Elevated temperature |  |  |
| Shaking / Mixing |  |  |
| Centrifugation |  |  |
| Use of Sharps |  |  |
| High Pressure |  |  |
| Other |  |  |

2.3 EMERGENCY PROCEDURES

Note: Do not cut and paste information on emergency procedures and first aid from the MSDS – it must reflect procedures that are available within the local area (laboratory, building or campus concerned) and be proportional to the extent of exposure anticipated.

2.3.1 Spillage or release

Describe the procedures in place for a spillage / release:

Within the laboratory but outside any primary containment facility such as a fume cupboard: Wear gloves. Mop up with paper towels, and store in aplastic bag for disposal Within a fume cupboard (if relevant): Mop up with clean room wipe or paper towel and clean off with acetone. Outside the laboratory e.g. en route to another part of the building / site:

2.3.2 First aid

Describe the local first aid arrangements that are in place for accidental exposure to any of these substances.

| If exposed please flush affected areas with plenty of water. Seek medical advice if necessary. First aid box and eye wash is available in the lab / cleanroom. |  |  |  |
| --- | --- | --- | --- |
|  | Mains fed eyewash / shower available | Eye wash bottles | Alternative eyewash facilities (describe) |

Chemical Weapons Act: http://www.opsi.gov.uk/Acts/acts1996/ukpga_19960006_en_1

Anti-terrorism, Crime and Security Act: http://www.legislation.gov.uk/ukpga/2001/24/contents

Drug Precursors: http://www3.imperial.ac.uk/safety/subjects/drugprecursors