# Basic Laboratory Training

## General

- Lab coats, safety spectacles (in required areas) and sensible footwear to be worn at all times when working in the laboratory

- No lone working

- No food or drink to be consumed in the laboratory

- Refer to “Building and Laboratory Access” at http://www3.imperial.ac.uk/mechanicalengineering/intranet/safety

Workshop Practice

- Power tools to be used only by authorised (by the Lab Technician) personnel who have undertaken the ME Workshop Course

- Work must not be undertaken if the Lab technician is not present.

Test Equipment

- Researchers must undertake individual equipment training and complete risk assessments before experimental work commences.

Solvent Usage

Organic solvents are commonly used to clean test specimens/rigs.

- Appropriate face masks and protective gloves  must be used at all times when handling solvents

- COSHH forms must be completed before use.

- All used solvents must be placed in appropriate waste plastic containers in the chemical laboratory

- All solvents must be stored in sealed containers

- Solvent bottles to be stored in metal cupboards or when open in the fume cupboard

- Minimum amount of solvent must be used to clean test devices, use only in well-ventilated room.  Solvents stored in wash bottles should be kept to a minimum and clearly labelled (solvent type/name of user).

- After use solvent soaked cleaning tissue to be placed in fume cupboard container until solvent has evaporated.  Dry tissue then placed in paper waste containers.

Chemicals/COSHH forms

- COSHH forms (see Imperial web xx) must be completed for all chemicals used in the laboratory and before chemicals are ordered.

- To order chemicals: submit order and COSHH form to Supervisor who will instruct the Lab. Secretary to place the order.  Chemical orders will NOT be accepted directly from researchers.

- Printed copies of the COSHH form to be kept by the researcher and given to the Lab. Technician for safety records

Chemicals ordered for specific users

- Guidelines for Solvent usage (when applicable) and chemicals/COSHH form apply.

- For chemical ordered for a specific user, the name of the user would be written on the bottle.

- The specific user assumes responsibility of storing and handling of the chemical in accordance with its MSDS and in such a way that will not cause inconvenience and harm to other users in the lab.

- When other sealed container is used for chemical storage, the name of the chemical, the name of the user and date should be clearly marked.

- For water, air, light, time-sensitive chemicals, the date of opening the chemical should be clearly marked on the bottle.

## Laser Safety

- Laser safety course and eye tests must be completed before work commences.

- Specialist equipment training (Dr Janet Wong) must be completed before work commences

Liquid Nitrogen Handling

- Researchers must undertake Cryogenic Liquid Handling Course before work commences

- Individual risk assessments must be completed

- Protective clothing (face mask, insulated gloves) must be used when handling

- Liquid nitrogen transportation only to be undertaken by the Lab. Technician.

Gas Bottles

- Must only be used in approved areas

- Must be secured firmly to the benches using appropriate fixing methods by the Lab. Technician

- COSHH forms must be completed

|  | I have undertaken this basic laboratory training. I have also read and understood this document. |
| --- | --- |

Laboratory Safety Introduction training undertaken by:

Researcher:					Signed:				Date:

Instructor:					Signed:				Date: